Mini Project Report

RADAR BASED OBJECT DETECTION SYSTEM

1. Introduction

Radar systems are widely used in military, aviation, autonomous vehicles, weather forecasting, and security applications for object detection and distance measurement.

In this mini project, a small-scale Radar Object Detection System is implemented using:

- Ultrasonic Sensor (HC-SR04) – measures distance using sound waves

- Servo Motor – rotates the sensor like real radar

- Arduino – processes measurements

- Processing Software – displays real-time radar visualization

The system detects nearby objects, calculates distance, and displays them in real-time radar format on a PC screen.

2. Objective

- To design a low-cost radar model using ultrasonic sensor and Arduino.

- To measure distance of objects from the sensor.

- To visualize detected objects on a radar-like graphical interface.

- To simulate real radar scanning using servo rotation.

3. Components Used

| Component | Quantity | Description |
| --- | --- | --- |
| Arduino UNO / Nano | 1 | Main microcontroller |
| Ultrasonic Sensor (HC-SR04) | 1 | Measures distance using ultrasonic waves |
| Servo Motor (SG90) | 1 | Rotates sensor from 0° to 180° |
| Jumper Wires | As required | Connections |
| USB Cable | 1 | Arduino programming + power |
| Laptop with Processing software | 1 | Radar display interface |

4. Working Principle

The radar works based on ultrasonic time-of-flight measurement.

Steps:

- The ultrasonic sensor sends a trigger pulse.

- Sound waves travel, hit an object, and reflect back.

- The sensor measures the echo time.

- Arduino calculates the distance using:

- The servo rotates the sensor from 0° to 180°.

- At each angle, distance is recorded.

- Processing software reads values from Arduino and displays them as radar dots.

5. Software Used

| Software | Purpose |
| --- | --- |
| Arduino IDE | Programming microcontroller |
| Processing IDE | Radar Visualization Interface |

6. Result and Output

- The radar successfully scans the environment from 0° to 180°.

- Objects are detected up to 2 meters (distance displayed in meters).

- Detected objects show colored indication:

  - 🟢 Green – close object

  - 🟡 Yellow – medium distance

  - 🔴 Red – far object

- A live radar animation runs on computer screen showing object location.

7. Applications

- Obstacle detection in robotics

- Blind spot monitoring

- Parking assistance systems

- Autonomous vehicle sensing

- Security surveillance

8. Advantages

- Low-cost implementation

- Accurate short-range detection

- Real-time monitoring

- Easy to modify and extend

9. Future Scope

- Replace ultrasonic sensor with LIDAR for higher range

- Wireless communication (WiFi/LoRa) for remote monitoring

- AI-based object recognition

- Waterproof outdoor version

10. Conclusion

This mini project successfully demonstrates a functional radar system capable of detecting objects and displaying their position visually. It provides a practical understanding of sensing, signal processing, and visualization concepts used in real-world radar systems.

Submitted by:

Name: Nishkarsh Tripathi, Shashank Pandey, Uday Chauhan, Rishi Pratap Singh, Shubham Singh
Project Title: Radar Object Detection System Using Arduino
Course/Department:  B.TECH / CSE-IOT
College: Raj Kumar Goel Institute Of Technology, Ghaziabad,Uttar Pradesh